MongoDB Replica Set Setup on Windows

Step-by-Step Guide  |  Date: 10 June 2026

# Prerequisites

An existing MongoDB 4.4 instance is already running on the system with the following configuration:

| Parameter | Value |
| --- | --- |
| Binary Location | C:\Program Files\MongoDB\Server\4.4\bin |
| --dbpath | C:\Program Files\MongoDB\Server\4.4\data\db |
| --logpath | C:\Program Files\MongoDB\Server\4.4\log\mongod.log |
| --port | 27017 |
| --storageEngine | wiredTiger |

ℹ Open all Command Prompt windows as Administrator (right-click > Run as administrator) to avoid access-denied errors.

# Replica Set Overview

Three mongod instances will form one replica set named prodrep:

| Node | Role | Port | dbpath | replSet |
| --- | --- | --- | --- | --- |
| Node 0 | PRIMARY | 27017 | C:\Program Files\MongoDB\Server\4.4\data | prodrep |
| Node 1 | SECONDARY | 27020 | c:\data1\db | prodrep |
| Node 2 | SECONDARY | 27030 | c:\data2\db | prodrep |

# Phase 1 — Create Directory Structure

## Step 1 — Create folders for both slave nodes

Open Command Prompt as Administrator and run:

mkdir c:\data1\config  c:\data1\db  c:\data1\log

mkdir c:\data2\config  c:\data2\db  c:\data2\log

# Phase 2 — Create Configuration Files

## Step 2a — Config file for Slave Node 1 (port 27020)

notepad c:\data1\config\mongod.cnf

Paste the following content and save:

dbpath=c:\data1\db

logpath=c:\data1\log\mongod.log

port=27020

storageEngine=wiredTiger

replSet=prodrep

## Step 2b — Config file for Slave Node 2 (port 27030)

notepad c:\data2\config\mongod.cnf

Paste the following content and save:

dbpath=c:\data2\db

logpath=c:\data2\log\mongod.log

port=27030

storageEngine=wiredTiger

replSet=prodrep

# Phase 3 — Start All mongod Instances

## Step 3 — Stop the default MongoDB service

Open services.msc and stop the MongoDB service, or run from an elevated prompt:

net stop MongoDB

⚠ Note: If port 27017 remains in use, check with: netstat -ano ¦ findstr :27017 and kill the PID via Task Manager.

## Step 4 — Start Primary Node (Terminal 1)

Open a new Administrator Command Prompt and run:

mongod --dbpath "C:\Program Files\MongoDB\Server\4.4\data"

--logpath "C:\Program Files\MongoDB\Server\4.4\log\mongod.log"

--port 27017 --storageEngine=wiredTiger

--journal --replSet prodrep

Leave this window open. Expected output:

## Step 5 — Start Slave Node 1 (Terminal 2)  &  Slave Node 2 (Terminal 3)

Open two more Administrator Command Prompt windows:

// Terminal 2 — Slave Node 1

mongod --dbpath "c:\data1\db"

--logpath "c:\data1\log\mongod.log"

--port 27020 --storageEngine=wiredTiger

--journal --replSet prodrep

// Terminal 3 — Slave Node 2

mongod --dbpath "c:\data2\db"

--logpath "c:\data2\log\mongod.log"

--port 27030 --storageEngine=wiredTiger

--journal --replSet prodrep

# Phase 4 — Initialize the Replica Set

## Step 6 — Connect to Primary and initialize

Open a 4th Command Prompt window (does not need to be Administrator):

mongo --port 27017

Once inside the mongo shell, define the replica set config and initiate:

rsconf = { _id: "prodrep", members: [ { _id: 0, host: "localhost:27017" } ] }

rs.initiate(rsconf)

After pressing Enter twice, the prompt changes to  prodrep:PRIMARY>

## Step 7 — Add Slave Nodes to the Replica Set

From the  prodrep:PRIMARY>  prompt, run:

rs.add("localhost:27020")

rs.status()

rs.add("localhost:27030")

rs.status()

The rs.status() output should show one PRIMARY and two SECONDARY members, all with health: 1.

# Phase 5 — Enable Reads on Secondary Nodes

## Step 8 — Connect to Slave 1 (port 27020) and enable reads

mongo --port 27020

rs.slaveOk()

show dbs

## Step 9 — Connect to Slave 2 (port 27030) and enable reads

mongo --port 27030

rs.slaveOk()

show dbs

⚠ Note: rs.slaveOk() is required on each secondary shell to allow read queries. Without it, queries on secondary nodes return an error.

# Phase 6 — Test Replication

## Step 10 — Insert data on Primary

From the  prodrep:PRIMARY>  prompt:

use repdb1

db.student.insert({ "name": "Avi" })

show dbs

db.student.find()

Expected: repdb1 appears in show dbs and the inserted document is returned.

## Step 11 — Verify replication on both Secondary nodes

Switch to each secondary shell and confirm the database and document were replicated:

// On port 27020 shell

show dbs

use repdb1

db.student.find()

// On port 27030 shell

show dbs

use repdb1

db.student.find()

Both secondaries must display repdb1 in show dbs and return { "name": "Avi" } from db.student.find(). This confirms successful replication.

# Summary

- Created directory structure for two slave nodes under c:\data1 and c:\data2.

- Created mongod.cnf configuration files for each slave node.

- Stopped the default MongoDB service on port 27017.

- Started all three mongod instances with --replSet prodrep.

- Initialized the replica set with rs.initiate() and added both slave nodes with rs.add().

- Enabled reads on secondary nodes with rs.slaveOk().

- Confirmed successful replication by inserting data on the primary and verifying on both secondaries.

ℹ Replication provides high availability: if the primary node goes down, one secondary is automatically elected as the new primary and operations continue without manual intervention.